Table of Contents

Standard Audio/Video Frame Packaging Format Definition	2

Revision History	2

1. Overview	3

1.1 Background	3

1.2 Document Scope	3

1.3 Data Description	3

1.4 Frame Data Strategies	3

1.4.1 Time Filling	3

2. Frame Packaging Format	4

2.1 Frame Package Header Structure	4

Stream Checksum	5

2.2 Extension Data	6

2.2.1 Video Information	7

2.2.2 RTC Time Information	7

2.2.3 Virtual Time Information	9

2.2.4 Integer (4-byte) List	9

2.2.5 Audio Information	10

2.2.6 Video Status	11

2.2.7 H.264 Information	11

2.2.8 Video Reference Frame Information	13

2.2.9 Video Encoding Information	14

2.2.10 Additional Data Type	14

2.2.11 GPS Data Type	14

2.2.12 Encoding Extension Information	17

2.2.13 AIBOX Algorithm Overlay Information Extension Data	17

2.2.14 Data Encryption and Tamper-Proofing Extension Data	18

2.2.15 Industry Extension Information	18

2.2.16 Customized Data Encryption and Tamper-Proofing Extension Information	21

3. Appendix	22

3.1 Data Types	22

3.2 Abbreviations	22

3.3 Change Log	22

Standard Audio/Video Frame Packaging Format Definition

Audio/Video Frame Packaging Format Definition

| Item | Value |
| --- | --- |
| Scope | Internal document |
| Confidentiality level | Top Secret |
| Created on | 2011-12-14 |
| Last modified on | 2026-04-30 |
| Version | 1.00.00 |

Shenzhen Streaming Video Technology Co., Ltd.

All rights reserved.

## Revision History

Revision notes: C = Created, A = Added, M = Modified, D = Deleted

| Date | Revision | Description | Author |
| --- | --- | --- | --- |
| 2018-05-14 | A | Added AES encryption key length field (192, 256) to RMFI2_VIDEOSTATE. |  |
| 2018-07-27 | A | Added extension field: RMFI2_VIDEOAIINFO. |  |
| 2018-08-22 | M | Modified AES encryption type for compatibility with legacy devices and platforms. |  |
| 2025-10-22 | A | Added Sections 2.2.14 to 2.2.16. |  |
| 2026-04-30 | A | Added OSD and mosaic information in Section 2.2.15. |  |

# 1. Overview

## 1.1 Background

To standardize and unify the work of the same platform and accelerate project development, this document defines the audio/video packaging format in order to maximize future compatibility.

In the first-generation standard format, bit flags were used to indicate the presence of each type of information. This approach had limitations: it made parsing cumbersome, and parsing became even more difficult as additional information was expanded.

The format defined in this document is called the second-generation standard format. It uses a looser organization method that is easier to extend and parse.

## 1.2 Document Scope

This document specifies not only the framework structure of the audio/video frame packaging format, but also the specific type information and related content.

Note: This document specifies only the packaging format of a single video frame, not the encoding format.

## 1.3 Data Description

All reserved fields must be filled with 0, meaning they must be completely cleared.

Please note that all members whose names contain “Virtual” represent virtual members rather than actual existing members. They are replaced by another entity.

## 1.4 Frame Data Strategies

In principle, according to the structure defined in this document, many fields in the video frame header may be selected flexibly. However, due to actual application requirements, some important information cannot be too arbitrary, such as absolute time. The following strategies are listed here. Unless there is a special reason, every program module should follow these strategies to support actual applications.

### 1.4.1 Time Filling

Absolute time is required in many places, so the following practices are recommended:

- Write absolute time in every I-frame.

- Write absolute time whenever the second changes.

During seeking, comparison is performed based on absolute time. This is especially true in the future when the concept of files is completely abandoned. Most customers require seek accuracy to be at least precise to the second. If a recording file does not guarantee one absolute time value per second, seeking becomes very troublesome and requires many calculations.

In actual playback, in order to align the current playback progress on the timeline with the time shown on the video image, it is recommended to write absolute time whenever the time changes. However, under the current condition of one I-frame per second, writing absolute time only in I-frames is basically accepted by customers. Therefore, this strategy may be treated as a future consideration and may not need to be implemented for the time being.

Writing absolute time is also required because the start time of a recording file may not be accurate for many possible reasons, such as program issues or alarm pre-recording. Since the absolute time in the stream cannot be correctly calculated from other information, these strategies must be followed.

Note: The timestamp in the frame header has no relationship with absolute time. It is only a counter generated according to a certain time precision. Because of disk or word-length limitations, it returns to 0 and recalculates after reaching a certain value. After each device power-off and restart, this counter generates a random starting number. The starting values generated across multiple startups are unrelated.

# 2. Frame Packaging Format

The overall frame format is as follows:

| Order | Component |
| --- | --- |
| 1 | Frame header structure, 12 bytes |
| 2 | Extension data, which may be empty |
| 3 | Audio/video data |

## 2.1 Frame Package Header Structure

The frame header structure is defined as follows:

struct RMSTREAM_HEADER
{
    unsigned long lFrameType;
    /* I-frame: x2dc; P-frame: x3dc; audio frame: x4dc; MJPEG frame: x7dc; ... */

    unsigned long lFrameLen:24;
    /* Length of video frame data only */

    unsigned long lStreamExam:8;
    /* Stream checksum */

    unsigned long lExtendLen:24;
    /* Length of extension data only */

    unsigned long lExtendCount:8;
};

Description:

lFrameType: Frame type identifier. I-frame is x2dc, P-frame is x3dc, and audio frame is x4dc, where x indicates the channel number. 0x00 is channel 1, 0x01 is channel 2, 0x02 is channel 3, …, 0x0f is channel 16, 0x10 is channel 17, and so on. For example, 02dc (0x63643200) indicates an I-frame of channel 1.

lFrameLen: Length of audio/video data only.

lStreamExam: Stream checksum, excluding this header structure and extension data. To reduce computation, this field defines that approximately 128 bytes of data are checked. See the following description.

lExtendLen: Length of extension data only. If there is no extension data, fill with 0.

lExtendCount: Number of extension data information entries. Since this uses a single byte, the number of entries is limited to 255.

Frame structure:

Component

RMSTREAM_HEADER

Extension data

Stream data (audio/video data)

### Stream Checksum

To ensure that the stream following the frame header is correct, a stream checksum is added here. Note that “stream” refers to the audio/video data. If the entire stream is checked, the workload is relatively large. Therefore, this document defines that only approximately 128 bytes in the stream are checked. If the stream itself is no longer than 128 bytes, all bytes are checked. If it is longer than 128 bytes, a uniform interval is used to select the bytes that actually need to be checked. The number of bytes to be checked, count, is calculated as follows, where len is the actual stream length:

ev = len / 128;
count = len / ev;

ev is the checksum interval.

The data to be checked is selected starting from the end of the stream until enough bytes have been selected. This strategy prevents the checksum from focusing on the beginning of the data when the stream is relatively small.

The logic for selecting checksum data is as follows, where char *p points to the start position of the stream and len is its length:

p += len - 1;
// Make p point to the last byte of the stream.

lStreamExam = 0;
for (i = 0; i < count; i++)
{
    lStreamExam += p[0];
    p -= ev;
}

## 2.2 Extension Data

The number of extension data types must not exceed 255.

Extension data starts with a unified header structure, defined as follows:

struct RMS_INFOTYPE
{
    unsigned long lInfoType:8;
    unsigned long lInfoLength:24;
};

Description:

lInfoType: Information type. Refer to the descriptions below. In each specific information structure below, the value of this field is identified by “Information Type”.

lInfoLength: Length of this information, including the length of this structure.

Extension data is stored in the following format:

Extension data layout

RMS_INFOTYPE + content of information 1

RMS_INFOTYPE + content of information 2

RMS_INFOTYPE + content of information 3

…

The lInfoType values are defined as follows:

| Value | Information type |
| --- | --- |
| 1 | Video information |
| 2 | RTC time information |
| 3 | Virtual time information |
| 4 | Integer (4-byte) list |
| 5 | Audio information |
| 6 | Video status |
| 7 | H.264 information |
| 8 | Video reference frame information |
| 9 | Video encoding information |
| 10 | Additional data information |
| 11 | GPS data type |
| 12 | Encoding extension information |
| 13 | AIBOX algorithm overlay information extension data |
| 14 | Data encryption and tamper-proofing extension data |
| 15 | Industry extension information |
| 16 | Customized data encryption and tamper-proofing extension information |

### 2.2.1 Video Information

Information type: 1

struct RMFI2_VIDEOINFO
{
    RMS_INFOTYPE stuInfoType;

    unsigned long lWidth:12;
    unsigned long lHeight:12;
    unsigned long lFPS:8;
};

Description:

stuInfoType: Information identifier.

lWidth: Video width.

lHeight: Video height.

lFPS: Frame rate, in frames per second.

This information is generally used only in video frames. In audio frames and black box files, this information is generally not required.

### 2.2.2 RTC Time Information

Information type: 2

struct RMFI2_RTCTIME
{
    RMS_INFOTYPE stuInfoType;
    RMS_DATETIME stuRtcTime;
};

Description:

stuInfoType: Information identifier.

stuRtcTime: Real time.

In the following two cases, this information must be written into video frames:

- I-frames.

- When the second changes.

The date-time structure is defined as follows:

struct RMS_DATETIME
{
    BYTE cYear;
    BYTE cMonth;
    BYTE cDay;
    BYTE cHour;
    BYTE cMinute;
    BYTE cSecond;

    unsigned short usMilliSecond:10;
    unsigned short usWeek:3;
    unsigned short usReserved:2;
    unsigned short usMilliValidate:1;
};

typedef struct RMS_DATETIME RMFI_DATETIME;

Description:

cYear: Year minus 2000. For example, 11 indicates 2011, 5 indicates 2005, and 113 indicates 2113.

cMonth: Month. Range: 1 to 12.

cDay: Day. Range: 1 to 31.

cHour: Hour. Range: 0 to 23.

cMinute: Minute. Range: 0 to 59.

cSecond: Second. Range: 0 to 59.

usMilliSecond: Millisecond. Range: 0 to 999.

usWeek: Weekday. Range: 1 to 7, representing Monday through Saturday and Sunday respectively. 0 indicates that the weekday field is invalid.

usMilliValidate: Indicates whether the millisecond field usMilliSecond is valid. 0 = invalid; 1 = valid.

### 2.2.3 Virtual Time Information

Information type: 3

struct RMFI2_VTIME
{
    RMS_INFOTYPE stuInfoType;
    ULONGLONG llVirtualTime;
};

Description:

stuInfoType: Information identifier.

llVirtualTime: Virtual time, in milliseconds. It is used only for internal program timing operations and does not need to be processed or displayed by the upper layer. Its precision must meet playback requirements. Within the same device, this virtual time only increases and never decreases; that is, it must remain monotonically increasing.

This information must be written in all frames, including video frames and audio frames.

### 2.2.4 Integer (4-byte) List

Information type: 4

struct RMFI2_LONGLIST
{
    RMS_INFOTYPE stuInfoType;

    unsigned char cItemCount;
    char reserved[3];

    RMFI2_LONGINFO *pstuVirLongList;
};

Description:

stuInfoType: Information identifier.

cItemCount: Number of elements, namely the number of elements in pstuVirLongList.

pstuVirLongList: Element list. In the actual structure, it is replaced by the actual list.

The list element is defined as follows:

struct RMFI2_LONGINFO
{
    unsigned char cType;
    char reserved[3];

    unsigned long lValue;
};

Description:

cType: Type of value. Refer to the definitions below.

lValue: Specific value.

Value type (cType) definitions:

| Value | Description |
| --- | --- |
| 1 | Distance from the start position of the previous I-frame to the start position of the current frame, in bytes. Multiple audio frames and P-frames may be included between them. If there is no previous I-frame, set lValue to 0. |

### 2.2.5 Audio Information

Information type: 5

struct RMFI2_AUDIOINFO
{
    RMS_INFOTYPE stuInfoType;

    unsigned long lPayloadType:4;
    unsigned long lSoundMode:1;
    unsigned long lPlayAudio:1;
    unsigned long lBitWidth:6;
    unsigned long lSampleRate:18;
    unsigned long lReserve:2;
};

Description:

lPayloadType: Audio encoding protocol type. 0 = ADPCM; 1 = G.726. For G.726, the bit rate uses ucAudioEncoderBps in RMFI2_ENCODEREXTENDINFO; if this field does not exist, the default is MEDIDA-16K. 2 = G.711A; 3 = G.711U; 4 = AMR; 5 = AAC; 6 = LPCM.

lSoundMode: Audio channel mode. 0 = mono; 1 = stereo.

lPlayAudio: Whether the audio frame should be played. 0 = do not play; 1 = play.

lBitWidth: Audio sampling precision: 8 (8-bit), 16 (16-bit).

lSampleRate: Sampling rate: 8000 (8K), 16000 (16K).

### 2.2.6 Video Status

Information type: 6

struct RMFI2_VIDEOSTATE
{
    RMS_INFOTYPE stuInfoType;

    unsigned long lVideoLost:1;
    unsigned long lRefType:3;       /* 0-1X; 1-2X; 3-4X; 4-custom */
    unsigned long lEncryptType:3;   /* 0x0-no encrypt; 0x1-aes; others-not defined */
    unsigned long lAesType:2;       /* 0:AES_192, 1:AES_128, 2:AES_256; others-not defined */
    unsigned long lReserve:20;
    unsigned long lCameraAngle:2;   /* 0x0-no angle; 0x1-+90; 0x2--90; others-not defined */
    unsigned long lCameraMirror:1;  /* 0x0-close; 0x1-open */
} __attribute__((packed));

Description:

lVideoLost: Video loss status. 0 = no video loss; 1 = video loss.

lRefType: Skip-frame encoding mode. 0 = 1x skip-frame encoding; 1 = 2x skip-frame encoding; 3 = 4x skip-frame encoding; 4 = custom.

lEncryptType: Encryption type. 0 = not encrypted; 0x1 = AES encryption; others are undefined.

lAesType: AES encryption key length. 0 = AES-192; 1 = AES-128; 2 = AES-256; others are undefined.

lReserve: Reserved bits. It is recommended to use them from back to front.

lCameraAngle: Camera installation angle. 0 = no angle; 1 = +90 degrees; 2 = -90 degrees; others are undefined.

lCameraMirror: Whether camera mirroring is enabled. 0 = disabled; 1 = enabled.

### 2.2.7 H.264 Information

Information type: 7

struct RMFI2_H264INFO
{
    RMS_INFOTYPE stuInfoType;

    unsigned long lPicBytesNum;
    unsigned long lPSkipMbNum;
    unsigned long lIpcmMbNum;
    unsigned long lInter16x8MbNum;
    unsigned long lInter16x16MbNum;
    unsigned long lInter8x16MbNum;
    unsigned long lInter8x8MbNum;
    unsigned long lIntra16MbNum;
    unsigned long lIntra8MbNum;
    unsigned long lIntra4MbNum;
    unsigned long lRefSliceType:8;
    unsigned long lRefType:8;
    unsigned long lReserve:16;
    unsigned long lUpdateAttrCnt;
} __attribute__((packed));

Description:

lPicBytesNum: Number of bytes used to encode the current frame.

lPSkipMbNum: Number of macroblocks in the current frame encoded in SKIP mode.

lIpcmMbNum: Number of macroblocks in the current frame encoded in IPCM mode.

lInter16x8MbNum: Number of macroblocks in the current frame using Inter16x8 prediction mode.

lInter16x16MbNum: Number of macroblocks in the current frame using Inter16x16 prediction mode.

lInter8x16MbNum: Number of macroblocks in the current frame using Inter8x16 prediction mode.

lInter8x8MbNum: Number of macroblocks in the current frame using Inter8x8 prediction mode.

lIntra16MbNum: Number of macroblocks in the current frame using Intra16 prediction mode.

lIntra8MbNum: Number of macroblocks in the current frame using Intra8 prediction mode.

lIntra4MbNum: Number of macroblocks in the current frame using Intra4 prediction mode.

lRefSliceType: Indicates the skip-frame reference mode under which the current encoded frame is a reference frame. 1 = 1X; 2 = 2X; 5 = 4X; other values are invalid.

lRefType: Reference frame type:

| Value | Description |
| --- | --- |
| 0 | IDR frame in the base layer. |
| 1 | P-frame in the base layer, used as a reference for other frames in the base layer. |
| 2 | P-frame in the base layer, used as a reference for frames in the enhancement layer. |
| 3 | P-frame in the enhancement layer, used as a reference for other frames in the enhancement layer. |
| 4 | P-frame in the enhancement layer, not used for reference. |

lReserve: Reserved field.

### 2.2.8 Video Reference Frame Information

Information type: 8

struct RMFI2_VIDEOREFINFO
{
    RMS_INFOTYPE stuInfoType;

    unsigned long lEnablePred:2;
    /* Indicates whether frames in the base layer are used as references by other frames in the base layer. */

    unsigned long lRefBaseInterval:8;
    unsigned long lRefEhanceInterval:8;
    unsigned long lRefSliceType:8;
    unsigned long lRefType:8;
    unsigned long lFrameId:8;
    unsigned long lReserve:22;
} __attribute__((packed));

Description:

lRefBaseInterval: Period of the base layer.

lRefEhanceInterval: Period of the enhancement layer.

lRefSliceType: Indicates which skip-frame encoding mode the frame is a reference frame for. 1 = reference frame in 1x skip-frame encoding; 2 = reference frame in 2x skip-frame encoding; 5 = reference frame in 4x skip-frame encoding; 6 = not a reference frame.

lRefType: Frame type in skip-frame encoding. 0 = IDR frame in the base layer; 1 = P-frame in the base layer, used as a reference for other base-layer frames; 2 = P-frame in the base layer, used as a reference for enhancement-layer frames; 3 = P-frame in the enhancement layer, used as a reference for other enhancement-layer frames; 4 = P-frame in the enhancement layer, not used for reference.

lFrameId: Frame number. I-frame is 0, and subsequent frames are counted sequentially.

lReserve: Reserved.

### 2.2.9 Video Encoding Information

Information type: 9

struct RMFI2_ENCODERINFO
{
    RMS_INFOTYPE stuInfoType;
    BYTE cEncoderType;
    BYTE cVideoType;
    BYTE Reserve[10];
} __attribute__((packed));

Description:

cEncoderType: Encoding mode: 0 = H.264; 1 = H.265; 2 = YUV data; 3 = MJPEG encoding.

cVideoType: Image format: 0 = YUV-420; 1 = YUV-422.

Reserve: Reserved.

### 2.2.10 Additional Data Type

Information type: 10

struct RMFI2_ADDITIONINFO
{
    RMS_INFOTYPE stuInfoType;

    char cReserved[8];
    char body[0];
} __attribute__((packed));

Description:

stuInfoType: Additional data type information.

body: Padding data used to ensure that the frame header and frame length are 32-byte aligned. The padding length is specified by stuInfoType; the default padding data is 0x0.

cReserve: Reserved.

### 2.2.11 GPS Data Type

struct RMFI_GPS_FRAME
{
    char cGpsStatus;              /* Whether GPS is valid */
    char cSpeedUnit;              /* Speed unit */
    unsigned short usSpeed;       /* Speed value */
    char cLatitudeDegree;         /* Latitude degrees */
    char cLatitudeCent;           /* Latitude minutes */
    char cLongitudeDegree;        /* Longitude degrees */
    char cLongitudeCent;          /* Longitude minutes */
    long lLatitudeSec;            /* Latitude seconds */
    long lLongitudeSec;           /* Longitude seconds */
    unsigned short usGpsAngle;    /* GPS angle */
    char cDirectionLatitude;      /* Latitude direction */
    char cDirectionLongitude;     /* Longitude direction */
    short sAltitude;              /* Height above sea level, signed value [-32768, 32767) */
    char reserved[2];
};

Description:

cGpsStatus: GPS validity flag. A indicates valid, V indicates invalid, and N indicates that no GPS module is connected.

cSpeedUnit: Speed unit. 0 = KM/kilometers; 1 = Mile/miles.

usSpeed: Speed value.

cLatitudeDegree: Latitude degrees, ASCII code, range 0 to 89.

cLatitudeCent: Latitude minutes, ASCII code, range 0 to 59.

cLongitudeDegree: Longitude degrees, ASCII code, range 0 to 179.

cLongitudeCent: Longitude minutes, ASCII code, range 0 to 59.

lLatitudeSec: Latitude seconds, in ten-millionths of a second, accurate to 7 decimal places after the seconds field.

lLongitudeSec: Longitude seconds, in ten-millionths of a second, accurate to 7 decimal places after the seconds field.

usGpsAngle: GPS angle, in hundredths of a degree. To maximize compatibility with previous structures, if the PC side finds that this value is greater than or equal to 360, it must divide the value by 100.

cDirectionLatitude: Latitude direction. N indicates north latitude, and S indicates south latitude.

cDirectionLongitude: Longitude direction. E indicates east longitude, and W indicates west longitude.

Description:

For latitude and longitude calculation, simply add the degree, minute, and second fields together, and then determine east/west longitude and north/south latitude according to the direction flag.

To accommodate future precision as much as possible, the GPS seconds value is stored as a long type with sufficient decimal places retained. Decimal places are retained by scaling the seconds value. Because the device side cannot use floating-point numbers, the seconds unit is one ten-millionth of a second. This provides precision to 7 decimal places after the decimal point, with a maximum value of 600,000,000 (60 x 10,000,000).

Because GPS information on the device side is generally stored in the format DDMM.MMMM, the following example shows how the device side can extract seconds and convert them into the format required here. Note: the specific calculation method depends on the data format provided by the GPS module.

Assume that the GPS value collected by the device side is 12158.3416:

- Degrees: 121

- Minutes: 58

- Seconds: 0.3416 minutes. Note that the unit is minutes.

Since seconds are in the decimal part and the unit is minutes, convert them to seconds first:

0.3416 * 60 = 20.496 seconds

Note: If 3416 * 60 is used instead, subsequent calculation will depend on the length of 3416. For example, if the value is 34164, it affects precision calculation, especially in a case such as 12158.0416, where the decimal part has a leading 0.

Then multiply by 10,000,000 to convert it to the precision required by this structure:

20.496 * 10,000,000 = 204,960,000 ten-millionths of a second

204960000 is the final value saved in the seconds field of this structure.

If seconds are calculated as an integer, the approximate process is as follows:

Assume the GPS value collected on the device side is abcde.fghi. Note that some of these letters may be 0.

- Degrees: abc

- Minutes: de

- Seconds: 0.fghi minutes

- Length of the seconds portion: 4

- Convert the seconds portion into an integer: fghi; after conversion, leading f, g, etc. may disappear if they are 0.

Multiply seconds by 60 to convert the unit to seconds:

fghi * 60 = X

Then multiply by 10,000,000 to convert to ten-millionths of a second:

X * (10,000,000 / 10000) = Y

Here, 10000 indicates that the length of the seconds portion is 4.

Overall conversion formula:

Y = 0.fghi * 60 * 10,000,000

Example formulas for converting the degree-minute-second fields in this structure:

Convert to seconds: s = Degree * 3600 + Cent * 60 + Sec * 1.00000 / 10,000,000;
Convert to minutes: c = Degree * 60 + Cent + Sec / 60.0000 / 10,000,000;
Convert to degrees: d = Degree + Cent / 60.000000 + Sec / 3600.0000 / 10,000,000;

struct RMFI2_GPSFRAMEINFO
{
    RMS_INFOTYPE stuInfoType;
    RMFI_GPS_FRAME stuGpsFrame;
    char reserved[8];
} __attribute__((packed));

### 2.2.12 Encoding Extension Information

Information type: 12

struct RMFI2_ENCODEREXTENDINFO
{
    RMS_INFOTYPE stuInfoType;

    unsigned char ucAudioEncoderBps;
    unsigned char ucReserved[11];
};

Description:

ucAudioEncoderBps: Audio encoding bit rate. It is used together with lPayloadType in RMFI2_AUDIOINFO:

When lPayloadType = G726: 0 = G726-MEDIDA-16K; 1 = G726-MEDIDA-24K; 2 = G726-MEDIDA-32K; 3 = G726-MEDIDA-40K; 4 = G726-16K; 5 = G726-24K; 6 = G726-32K; 7 = G726-40K.

When lPayloadType has another value, this field is invalid.

### 2.2.13 AIBOX Algorithm Overlay Information Extension Data

Information type: 13

struct RMFI2_VIDEOAIINFO
{
    RMS_INFOTYPE stuInfoType;

    unsigned long lReserve;
    unsigned long long ullPts;  // Algorithm frame PTS. Note: this is not the same as the PTS of this video frame.
    unsigned long lAIInfoLen;   // Algorithm overlay information data length.
    char body[0];               // Algorithm overlay information content.
} __attribute__((packed));

### 2.2.14 Data Encryption and Tamper-Proofing Extension Data

Information type: 14

When version = 0:

typedef struct tag_SecurityDataInfo
{
    xint32_t version;              // Key version.
    xuint8_t keyid[28];            // Unique identifier of the RSA key pair.
    xuint8_t encryptedAesKey[256]; // AES key encrypted by RSA.
    xuint8_t encryptedHash[16];    // Data hash encrypted by AES.
} SecurityDataInfo_t;

typedef struct
{
    RMS_INFOTYPE stuInfoType;
    SecurityDataInfo_t stuDataSecurity;
} RMFI2_DATASECURITY;

When version = 1:

typedef struct tag_SecurityDataInfoV2
{
    xint32_t version;              // Key version.
    xuint8_t keyid[28];            // Unique identifier of the RSA key pair.
    xuint8_t encryptedAesKey[256]; // AES key encrypted by RSA.
    xuint8_t encryptedHash[64];    // Data hash encrypted by AES.

    xuint32_t tamperProofAlg:4;    // Tamper-proofing algorithm: 0 = none; 1 = MD5; 2 = SHA-256; 3 = SHA-512.
    xuint32_t reserved0:28;        // Reserved extension bits, all 0.
    xuint32_t reserved1;           // Reserved extension field, all 0 by default.
} SecurityDataInfoV2_t;

typedef struct
{
    RMS_INFOTYPE stuInfoType;
    SecurityDataInfoV2_t stuDataSecurity;
} RMFI2_DATASECURITY_V2;

### 2.2.15 Industry Extension Information

Information type: 15

struct RMFI2_INDUSTRYEXDATA
{
    RMS_INFOTYPE stuInfoType; // Common header.
    xuint16_t dataLen;        // Body size.
    char cIndustryType;       // Business type, defined and maintained by the industry.
    char cReserve;            // Reserved.
    char body[0];             // Industry transparent-transmission data.
} __attribute__((packed));

#### 2.2.15.1 OSD and Mosaic Business Type

Currently used for overseas freight. cIndustryType = 2.

// Overseas freight industry extension data information.
typedef enum
{
    EXTEND_MOSIC = 0,        // Mosaic information. Overlay I-frames only.
    EXTEND_BASIC,            // Basic information. Overlay I-frames only.
    EXTEND_ALARM,            // Alarm information. Overlay I-frames only.
    EXTEND_CHNNAME,          // Channel name. Overlay I-frames only.
    EXTEND_VEHICLE,          // License plate number. Overlay I-frames only.
    EXTEND_REALTIME_DATA,    // Real-time data, etc. I-frames and P-frames.
    EXTEND_DATA_TYPE_MAX = 32
} eIndustryExtendDataType;

Overseas freight extension frame header information:

struct RMFI_INDUSTRY_EXTEND_FRAME_HEADER
{
    xuint8_t dataVer:4;      // Data version number. Default is 0; only 0 is processed.
    xuint8_t reserve:4;      // Reserved.
    xuint8_t dataCount;      // Number of data items in the body.
    xuint16_t dataLen;       // Body data length.
    xuint32_t dataTypeMask;  // Data type mask. Refer to @eIndustryExtendDataType.
    char body[0];            // Data.
};

Item header structure:

struct RMFI2_EXDATA_ITEM_HEADER
{
    xuint8_t dataType;   // Data type. See @eIndustryExtendDataType.
    xuint16_t length;    // Data length.
    xuint8_t reserve1;
} __attribute__((packed));

Mosaic information. If this information does not exist, treat it as not detected by the device:

struct EXDATA_MOSIC
{
    RMFI2_EXDATA_ITEM_HEADER dataHeader;

    xuint8_t aiEn:3;
    // Device AI masking: 0 = privacy not detected; 1 = privacy detected;
    // 2 = privacy detected + masked (reserved).

    xuint8_t reserve:5;  // Reserved.

    xuint8_t mosicType;
    // When aiEn = 1, indicates which information this algorithm has recognized.
    // BSD algorithm: bit0 = pedestrian; bit1 = vehicle.
    // Other algorithms: no mask flag is required. After aiEn is enabled,
    // all privacy targets are recognized.

    xint8_t reserve[3];  // Reserved.
} __attribute__((packed));

Basic item information:

struct EXDATA_BASIC
{
    RMFI2_EXDATA_ITEM_HEADER dataHeader;

    xuint32_t aiUse:10;     // Algorithm purpose. Refer to MDVR-MAIN-USE in the parameter document.
    xuint32_t caliState:3;  // Calibration status. 0 = not calibrated; 1 = calibrated.
    xuint32_t dst:2;        // Local daylight saving time offset, valid after zero time zone is enabled. Unit: 30 minutes.
    xuint32_t reserve:17;   // Reserved.

    xint8_t zone;           // Local time zone, valid after zero time zone is enabled. Unit: 15 minutes.
    xint8_t reserve1[3];    // Reserved.
} __attribute__((packed));

Alarm item information:

struct EXDATA_ALARM_ITEM
{
    xuint16_t mainId;   // Alarm main ID. Refer to the ALARMTYPE description in Section 3.4.1.5 of the N9M protocol.
    xuint16_t subId;    // Alarm sub ID. Refer to the additional data description of each alarm in Section 3.4.1.5 of the N9M protocol.
    xuint8_t length;    // Alarm string length. If the length is not 0, the terminal concatenates the alarm according to the alarm ID.
    xint8_t reserve[3]; // Reserved.
    char body[0];       // Alarm string.
} __attribute__((packed));

Alarm information:

struct EXDATA_ALARM
{
    RMFI2_EXDATA_ITEM_HEADER dataHeader;
    xuint8_t count;            // Number of alarm entries.
    xint8_t reserve[3];        // Reserved.
    EXDATA_ALARM_ITEM stuAlarmItem[0]; // Alarm item information.
} __attribute__((packed));

Channel name information:

struct EXDATA_CHNNAME
{
    RMFI2_EXDATA_ITEM_HEADER dataHeader;
    xbyte_t chnName[16];       // Channel name.
} __attribute__((packed));

License plate information:

struct EXDATA_VEHICLE
{
    RMFI2_EXDATA_ITEM_HEADER dataHeader;
    xbyte_t vehicle[32];       // License plate number.
    xbyte_t bid[32];           // Custom/internal number.
} __attribute__((packed));

Real-time information:

struct EXDATA_REALTIME_DATA
{
    RMFI2_EXDATA_ITEM_HEADER dataHeader;

    xuint16_t speed:9;    // Fused vehicle speed, in km/h.
    xuint16_t turn:2;     // Turn signal: 0 = no turn / not recognized; 1 = left turn; 2 = right turn; 3 = hazard lights.
    xuint16_t reserve:5;  // Reserved.

    xint8_t reserve1[2];  // Reserved.
} __attribute__((packed));

### 2.2.16 Customized Data Encryption and Tamper-Proofing Extension Information

Information type: 16

typedef struct tag_SecurityDataInfo
{
    xuint8_t encryptedAeskey[32]; // AES key encrypted with the AES-encrypted digest value of the device private key.
} SecurityDataInfo_t;

typedef struct
{
    RMS_INFOTYPE stuInfoType;
    SecurityDataInfo_t stuDataSecurity;
} RMFI2_DATASECURITY;

This is mutually exclusive with the Type 14 extension area. It is mainly used to remain compatible with the 1.0 design and is used under a specific encryption scheme.

# 3. Appendix

## 3.1 Data Types

The following data types and their lengths are defined:

| Data type | Length / description |
| --- | --- |
| int, unsigned int | 4-byte integer |
| short, unsigned short | 2-byte integer |
| char, unsigned char | 1-byte integer |
| long long | 8 bytes; __int64 in Windows, or another 8-byte numeric data type |

Byte order follows the Intel specification, meaning the high byte comes later.

## 3.2 Abbreviations

| Abbreviation | Meaning |
| --- | --- |
| VFI | Video Frame Information |
| FI | Frame Information |
| ACCEL | Acceleration |
| FIM | Frame Information Mode |

## 3.3 Change Log

- The length is lFrameLen. The checksum lStreamExam checks only part of this data.

- Fixed frame header structure, length 12 bytes.

- Extension data length is lExtendLen.